SIMATS ENGINEERING

ASSESSMENT TOOL 2: Scenario-Based

COURSE NAME: COMPUTER VISION		            COURSE CODE:	ITA0518

COURSE OUTCOME COVERED

CO2: Apply image enhancement and segmentation techniques for extracting meaningful regions from images. (BTL 3)

Assessment Tool Weightage: 20%

Scenario-Based Questions

1. Low Contrast Image

(a) Interpretation of the Scenario

A grayscale image appears dull because there is only a small difference between the intensity values of the object and the background. This results in poor visibility, making it difficult to distinguish image features. Low contrast usually occurs due to poor lighting conditions, limited dynamic range of the camera, or improper image acquisition.

(b) Key Issues Affecting Image Visibility

- Poor distinction between foreground and background.

- Narrow range of gray-level intensities.

- Loss of important image details.

- Difficulty in object detection and segmentation.

- Reduced visual quality and interpretability.

(c) Appropriate Enhancement Technique

Histogram Equalization (HE) is applied to improve image contrast.

Procedure:

- Calculate the histogram of the image.

- Compute the cumulative distribution function (CDF).

- Map old intensity values to new values using the CDF.

- Generate the enhanced image with improved contrast.

(d) Justification

Histogram Equalization redistributes pixel intensities over the entire grayscale range, thereby increasing contrast and revealing hidden details. It is simple, efficient, and suitable for grayscale images with low contrast.

(e) Conclusion

Histogram Equalization effectively enhances low-contrast images by improving the intensity distribution. The enhanced image provides better visibility, making it suitable for further image processing tasks such as segmentation and feature extraction.

2. Uneven Illumination

(a) Interpretation of the Scenario

The image contains bright and dark regions because of non-uniform lighting during image acquisition. Uneven illumination causes inconsistent brightness across the image, making object analysis difficult.

(b) Key Factors Causing Intensity Variation

- Non-uniform light source.

- Shadows cast by objects.

- Variations in surface reflectance.

- Camera exposure differences.

- Environmental lighting conditions.

(c) Suitable Enhancement Method

Apply Homomorphic Filtering.

Procedure:

- Convert the image to the logarithmic domain.

- Perform Fourier Transform.

- Apply a high-pass filter to suppress low-frequency illumination components.

- Perform inverse Fourier Transform.

- Convert back to the spatial domain.

(d) Justification

Homomorphic Filtering separates illumination and reflectance components. It reduces lighting variations while preserving object details, resulting in a uniformly illuminated image.

(e) Conclusion

Homomorphic Filtering effectively normalizes uneven illumination, improves brightness consistency, and enhances image quality for accurate analysis.

3. Salt-and-Pepper Noise

(a) Interpretation of the Scenario

Salt-and-pepper noise appears as randomly distributed white and black pixels in an image. It commonly occurs during image transmission, sensor malfunction, or scanning, reducing document readability.

(b) Key Issues Affecting Image Quality

- Random black and white pixels.

- Reduced readability of text.

- Distortion of fine details.

- Difficulty in segmentation and OCR.

- Lower image quality.

(c) Appropriate Filtering Technique

Apply the Median Filter.

Procedure:

- Select a sliding window (e.g., 3×3).

- Arrange neigh boring pixel values in ascending order.

- Replace the center pixel with the median value.

- Repeat for the entire image.

(d) Justification

The Median Filter removes impulse noise effectively without blurring edges. Unlike mean filtering, it preserves image details while eliminating isolated noisy pixels.

(e) Conclusion

Median Filtering efficiently removes salt-and-pepper noise while maintaining edge information, thereby improving document readability and image quality.

4. Blurred Edges

(a) Interpretation of the Scenario

The image becomes overly smooth after noise removal because excessive filtering removes high-frequency edge information along with noise. As a result, object boundaries become blurred.

(b) Key Issues

- Loss of edge sharpness.

- Reduced image details.

- Poor object boundary definition.

- Decreased segmentation accuracy.

- Difficulty in feature extraction.

(c) Appropriate Technique

Apply Unsharp Masking (Image Sharpening).

Procedure:

- Create a blurred version of the image.

- Subtract the blurred image from the original to obtain edge details.

- Add the extracted edges back to the original image.

- Obtain a sharpened image.

(d) Justification

Unsharp Masking enhances high-frequency components and restores edge clarity without significantly amplifying noise. It improves visual quality and boundary definition.

(e) Conclusion

Image sharpening restores blurred edges, making object boundaries clearer and improving the accuracy of subsequent image processing operations.

5. High-Frequency Noise

(a) Interpretation of the Scenario

The satellite image contains fine-grained random noise represented by rapid intensity variations. This high-frequency noise obscures important surface features.

(b) Key Issues Affecting Image Clarity

- Reduced visibility of land features.

- Distortion of image details.

- Difficulty in feature extraction.

- Reduced segmentation accuracy.

- Increased processing errors.

(c) Frequency Domain Filtering Method

Apply a Gaussian Low-Pass Filter in the frequency domain.

Procedure:

- Compute the Fourier Transform.

- Apply a Gaussian Low-Pass Filter.

- Remove high-frequency noise.

- Perform inverse Fourier Transform.

(d) Justification

The Gaussian Low-Pass Filter smoothly suppresses high-frequency noise without introducing ringing artifacts. It preserves important image structures while reducing noise.

(e) Conclusion

Gaussian Low-Pass Filtering improves satellite image clarity by removing fine-grained noise while preserving useful information for analysis.

6. Object Boundary Detection

(a) Interpretation of the Scenario

In medical images, accurate tissue boundary detection is essential for identifying organs, tumors, and abnormal structures. Clear boundaries help doctors perform accurate diagnosis and treatment planning.

(b) Key Challenges

- Noise in medical images.

- Low contrast between tissues.

- Weak or discontinuous edges.

- Complex anatomical structures.

- Variations in tissue intensity.

(c) Suitable Edge Detection Method

Apply the Canny Edge Detection Algorithm.

Steps:

- Smooth the image using a Gaussian filter.

- Compute image gradients.

- Apply non-maximum suppression.

- Perform double thresholding.

- Track edges using hysteresis.

(d) Justification

The Canny algorithm provides accurate edge localization, detects weak edges, suppresses noise, and minimizes false edge detection. It is widely used in medical image analysis.

(e) Conclusion

Canny Edge Detection accurately identifies tissue boundaries, improving medical image interpretation and supporting reliable clinical diagnosis.

7. Simple Segmentation

(a) Interpretation of the Scenario

The objects in the image have intensity values that are clearly different from the background. Therefore, segmentation can be performed by separating pixels based on their gray-level intensity.

(b) Key Features Enabling Separation

- High contrast between object and background.

- Distinct intensity values.

- Minimal overlap of gray levels.

- Uniform lighting conditions.

- Well-defined object boundaries.

(c) Suitable Segmentation Technique

Apply Global Thresholding.

Procedure:

- Select an appropriate threshold value.

- Assign pixels above the threshold to the object.

- Assign pixels below the threshold to the background.

- Generate the segmented binary image.

(d) Justification

Global Thresholding is computationally simple, fast, and highly effective when there is a clear intensity difference between the object and the background.

(e) Conclusion

Global Thresholding accurately separates objects from the background in simple images, providing reliable segmentation with minimal computational complexity.

8. Complex Segmentation

(a) Interpretation of the Scenario

The image contains multiple regions whose intensity values overlap, making it difficult to distinguish objects from the background using simple thresholding. Such situations are common in medical imaging, remote sensing, and natural scene analysis where different objects have similar gray-level intensities.

(b) Key Issues

- Overlapping intensity values.

- Poor contrast between neighboring regions.

- Ambiguous object boundaries.

- High possibility of incorrect segmentation.

- Presence of noise and texture variations.

(c) Appropriate Advanced Segmentation Method

Apply the Watershed Segmentation Algorithm with marker-controlled preprocessing.

Procedure:

- Preprocess the image using smoothing to reduce noise.

- Compute the image gradient.

- Identify foreground and background markers.

- Apply marker-controlled watershed segmentation.

- Extract the segmented regions.

(d) Justification

Marker-controlled Watershed Segmentation effectively separates touching or overlapping regions by using gradient information and predefined markers. It reduces over-segmentation and accurately identifies object boundaries even when intensity values overlap.

(e) Conclusion

Watershed Segmentation provides accurate separation of complex image regions with overlapping intensities, improving segmentation quality and enabling reliable image analysis.

9. Broken Edges

(a) Interpretation of the Scenario

After edge detection, the object boundaries appear discontinuous because some edge pixels are missing due to noise, weak gradients, or improper threshold selection. This results in incomplete object outlines.

(b) Key Issues

- Discontinuous object boundaries.

- Weak edge responses.

- Noise causing missing edge pixels.

- Poor connectivity between edge segments.

- Reduced accuracy in object recognition.

(c) Suitable Method

Apply Morphological Closing (Dilation followed by Erosion).

Procedure:

- Perform edge detection.

- Apply dilation to connect nearby edge segments.

- Apply erosion to restore the original edge thickness.

- Obtain continuous object boundaries.

(d) Justification

Morphological Closing effectively fills small gaps and connects broken edges without significantly altering object shapes. It improves boundary continuity and enhances segmentation performance.

(e) Conclusion

Morphological Closing restores continuous object boundaries, resulting in improved edge connectivity and more accurate object detection.

10. Combined Requirement

(a) Interpretation of the Scenario

An industrial inspection system requires removing image noise while preserving important edges so that defects such as cracks, scratches, or surface irregularities can be detected accurately.

(b) Key Issues

- Random image noise reduces visibility.

- Excessive smoothing may blur edges.

- Defect boundaries must remain sharp.

- High detection accuracy is required.

- Efficient preprocessing is essential.

(c) Appropriate Sequence of Techniques

- Apply a Median Filter to remove impulse noise.

- Apply a Bilateral Filter to smooth the image while preserving edges.

- Perform Canny Edge Detection to detect defect boundaries.

(d) Justification

The Median Filter removes noise effectively, the Bilateral Filter smooths homogeneous regions while preserving edges, and the Canny algorithm accurately detects object boundaries. This sequence ensures clean images with clear defect edges.

(e) Conclusion

The combination of Median Filtering, Bilateral Filtering, and Canny Edge Detection provides effective noise removal while preserving critical edges, making it suitable for industrial defect inspection.

11. Low Brightness Image

(a) Interpretation of the Scenario

The image is captured under low-light conditions, resulting in insufficient illumination. Most pixel values lie in the darker intensity range, making important image details difficult to observe.

(b) Key Issues

- Poor visibility.

- Loss of details in dark regions.

- Low contrast.

- Difficulty in feature extraction.

- Reduced image quality.

(c) Appropriate Enhancement Technique

Apply Gamma Correction (Gamma < 1).

Procedure:

- Normalize pixel values.

- Apply gamma transformation with γ < 1.

- Restore the enhanced image.

(d) Justification

Gamma Correction selectively brightens dark regions without excessively increasing brightness in already illuminated areas. It improves visibility while preserving image details.

(e) Conclusion

Gamma Correction enhances low-brightness images by increasing visibility and improving image quality for further processing.

12. Overexposed Image

(a) Interpretation of the Scenario

The image appears excessively bright because too much light reached the camera sensor during image acquisition. As a result, bright regions become saturated, causing loss of important image information.

(b) Key Issues

- Saturated pixel values.

- Loss of details in bright areas.

- Reduced image contrast.

- Difficulty in object identification.

- Poor intensity representation.

(c) Suitable Correction Technique

Apply Contrast Stretching (Intensity Normalization).

Procedure:

- Determine the minimum and maximum intensity values.

- Stretch the intensity values across the available grayscale range.

- Generate the corrected image.

(d) Justification

Contrast Stretching redistributes intensity values, reduces excessive brightness, and improves visibility of hidden details without introducing artifacts.

(e) Conclusion

Contrast Stretching effectively corrects overexposed images by restoring contrast and enhancing important image details.

13. Gaussian Noise

(a) Interpretation of the Scenario

Gaussian noise is a random additive noise that follows a normal (Gaussian) distribution. It commonly affects smooth image regions and reduces overall image quality.

(b) Key Issues

- Grainy appearance.

- Reduced image clarity.

- Distorted fine details.

- Lower signal-to-noise ratio.

- Difficulty in image analysis.

(c) Appropriate Smoothing Filter

Apply a Gaussian Filter.

Procedure:

- Create a Gaussian kernel.

- Convolve the image with the kernel.

- Produce the smoothed image.

(d) Justification

The Gaussian Filter effectively suppresses Gaussian noise while preserving important image structures better than simple averaging filters. It provides smooth and natural-looking results.

(e) Conclusion

Gaussian Filtering reduces random noise and improves image quality, making the image suitable for further processing.

14. Speckle Noise

(a) Interpretation of the Scenario

Speckle noise is a multiplicative noise commonly found in ultrasound, radar, and SAR images. It appears as granular patterns that reduce image clarity.

(b) Key Issues

- Grainy image texture.

- Reduced tissue visibility.

- Difficulty in boundary detection.

- Lower diagnostic accuracy.

- Distorted image features.

(c) Suitable Filtering Technique

Apply the Lee Filter.

Procedure:

- Estimate local mean and variance.

- Apply adaptive filtering based on local statistics.

- Preserve edges while reducing speckle noise.

(d) Justification

The Lee Filter is specifically designed for multiplicative speckle noise. It smooths noisy regions while preserving important edges and anatomical structures.

(e) Conclusion

The Lee Filter effectively reduces speckle noise, enhancing ultrasound image clarity and improving diagnostic accuracy.

15. Loss of Fine Details

(a) Interpretation of the Scenario

The image has lost fine details because excessive filtering removed high-frequency information along with noise. This reduces image sharpness and feature visibility.

(b) Key Issues

- Loss of texture.

- Reduced sharpness.

- Blurred edges.

- Missing small image features.

- Decreased analysis accuracy.

(c) Suitable Enhancement Technique

Apply Unsharp Masking (Image Sharpening).

Procedure:

- Generate a blurred version of the image.

- Subtract it from the original image.

- Add the resulting edge information back to the original image.

(d) Justification

Unsharp Masking enhances fine details and restores image sharpness while preserving overall image quality.

(e) Conclusion

Image sharpening restores lost details, improves edge visibility, and enhances the effectiveness of subsequent image processing tasks.

16. Edge Enhancement Requirement

(a) Interpretation of the Scenario

The application requires emphasizing object edges because edges contain important structural information used for feature extraction, object recognition, and image analysis.

(b) Key Issues

- Weak edge contrast.

- Noise affecting edge detection.

- Difficulty in locating object boundaries.

- Loss of important structural features.

- Reduced feature extraction accuracy.

(c) Appropriate Method

Apply the Laplacian Filter followed by image sharpening.

Procedure:

- Apply the Laplacian operator to detect intensity changes.

- Add the Laplacian image to the original image.

- Obtain the edge-enhanced image.

(d) Justification

The Laplacian Filter highlights regions with rapid intensity changes, making edges more prominent. It is computationally efficient and improves feature extraction performance.

(e) Conclusion

Laplacian-based edge enhancement produces sharper object boundaries, improving the accuracy of feature extraction, object recognition, and computer vision applications.

18. Threshold Selection Problem

(a) Interpretation of the Scenario

The image requires segmentation, but selecting a suitable threshold is difficult because the intensity values of the object and background overlap. Variations in illumination and noise make it challenging to choose a single threshold that accurately separates all regions.

(b) Key Issues in Threshold Selection

- Overlapping object and background intensities.

- Uneven illumination across the image.

- Presence of image noise.

- Incorrect threshold causing over- or under-segmentation.

- Reduced segmentation accuracy.

(c) Suitable Thresholding Method

Apply Otsu's Thresholding Method.

Procedure:

- Calculate the image histogram.

- Compute the variance between foreground and background classes.

- Automatically determine the optimal threshold.

- Segment the image into object and background.

(d) Justification

Otsu's method automatically selects the threshold by maximizing the variance between image classes. It eliminates manual threshold selection and provides accurate segmentation for images with two dominant intensity groups.

(e) Conclusion

Otsu's Thresholding improves segmentation accuracy by automatically selecting an optimal threshold, making it suitable for images where manual threshold selection is difficult.

19. Texture-Based Segmentation

(a) Interpretation of the Scenario

The image contains regions that differ mainly in texture rather than pixel intensity. Since neighboring regions may have similar gray-level values, intensity-based segmentation alone cannot accurately separate them.

(b) Key Issues with Intensity-Based Methods

- Similar intensity values in different regions.

- Failure to distinguish textured surfaces.

- Poor segmentation accuracy.

- Noise affecting texture information.

- Difficulty in identifying region boundaries.

(c) Appropriate Segmentation Approach

Apply Texture-Based Segmentation using GLCM (Gray Level Co-occurrence Matrix) Features.

Procedure:

- Compute GLCM texture features such as contrast, correlation, energy, and homogeneity.

- Extract texture feature vectors.

- Classify or cluster regions based on texture similarity.

- Generate the segmented image.

(d) Justification

GLCM captures texture information instead of relying only on intensity values. It effectively distinguishes regions with similar brightness but different surface patterns, producing more accurate segmentation.

(e) Conclusion

Texture-based segmentation using GLCM provides reliable separation of regions with different textures, improving segmentation performance in complex images.

20. Multiple Object Detection

(a) Interpretation of the Scenario

The image contains several objects having similar intensity values. Because the objects appear nearly identical in brightness, separating them from one another becomes difficult using simple segmentation methods.

(b) Key Issues in Distinguishing Objects

- Similar intensity values among objects.

- Objects may touch or overlap.

- Weak object boundaries.

- Increased possibility of incorrect segmentation.

- Difficulty in individual object identification.

(c) Suitable Segmentation Technique

Apply Marker-Controlled Watershed Segmentation.

Procedure:

- Remove image noise using preprocessing.

- Compute the gradient image.

- Select foreground and background markers.

- Apply the Watershed algorithm.

- Obtain separate object regions.

(d) Justification

Marker-Controlled Watershed Segmentation accurately separates adjacent or touching objects by using markers and gradient information. It minimizes over-segmentation and clearly identifies individual objects.

(e) Conclusion

Marker-Controlled Watershed Segmentation effectively separates multiple objects with similar intensity values, improving object detection accuracy.

21. Edge Noise Problem

(a) Interpretation of the Scenario

The detected edges are corrupted by noise, resulting in false edge pixels and discontinuous boundaries. Noise makes it difficult to distinguish true object edges from unwanted image artifacts.

(b) Key Challenges in Edge Detection

- Presence of random noise.

- False edge detection.

- Broken or weak edges.

- Reduced edge localization accuracy.

- Difficulty in object recognition.

(c) Suitable Solution

Apply a Gaussian Filter followed by Canny Edge Detection.

Procedure:

- Smooth the image using a Gaussian filter to reduce noise.

- Compute image gradients.

- Perform non-maximum suppression.

- Apply double thresholding and edge tracking.

- Obtain clean and continuous edges.

(d) Justification

The Gaussian filter removes noise before edge detection, while the Canny algorithm accurately detects strong and weak edges with minimal false responses. This combination improves edge quality and continuity.

(e) Conclusion

Gaussian smoothing followed by Canny Edge Detection effectively reduces edge noise, producing accurate and continuous boundaries suitable for image analysis.

22. Region Merging Requirement

(a) Interpretation of the Scenario

The image contains fragmented regions that actually belong to the same object but have been separated during segmentation. These fragmented regions need to be merged to obtain a complete and meaningful representation of the object.

(b) Key Challenges in Region Segmentation

- Fragmentation of a single object into multiple regions.

- Small gaps between neighboring regions.

- Noise causing false boundaries.

- Difficulty in identifying similar adjacent regions.

- Reduced segmentation accuracy.

(c) Appropriate Technique

Apply Region Merging Algorithm.

Procedure:

- Perform initial segmentation.

- Compare neighboring regions based on intensity, texture, or color similarity.

- Merge adjacent regions that satisfy the similarity criterion.

- Repeat until no further merging is possible.

(d) Justification

Region Merging combines neighboring regions with similar characteristics, reducing fragmentation while preserving object boundaries. It produces larger and more meaningful regions for image analysis.

(e) Conclusion

Region Merging effectively combines fragmented image regions, improving segmentation quality and providing complete object representation for further processing.

23. Over-Segmentation

(a) Interpretation of the Scenario

Over-segmentation occurs when an image is divided into too many small regions due to excessive sensitivity to noise, texture, or intensity variations. Instead of identifying meaningful objects, the algorithm creates many unnecessary segments.

(b) Key Issues Affecting Segmentation

- Excessive number of small regions.

- Noise creating false boundaries.

- Increased computational complexity.

- Difficulty in object recognition.

- Reduced segmentation accuracy.

(c) Suitable Correction Method

Apply Marker-Controlled Watershed Segmentation with image smoothing.

Procedure:

- Apply Gaussian smoothing to reduce noise.

- Generate foreground and background markers.

- Apply marker-controlled Watershed segmentation.

- Merge unnecessary small regions.

(d) Justification

Marker-controlled Watershed reduces false boundaries by guiding the segmentation process with predefined markers. It minimizes over-segmentation while preserving meaningful object regions.

(e) Conclusion

Marker-controlled Watershed Segmentation effectively reduces over-segmentation, resulting in accurate and meaningful image regions.

24. Under-Segmentation

(a) Interpretation of the Scenario

Under-segmentation occurs when different objects or regions are incorrectly merged into a single segment. This causes the loss of individual object identities and reduces segmentation accuracy.

(b) Key Issues Affecting Separation

- Similar intensity values between neighboring objects.

- Weak object boundaries.

- Insufficient edge information.

- Incorrect threshold selection.

- Loss of object details.

(c) Suitable Solution

Apply Marker-Controlled Watershed Segmentation or Edge-Based Segmentation.

Procedure:

- Detect object boundaries using edge detection.

- Generate accurate foreground markers.

- Apply Watershed segmentation.

- Separate merged regions into individual objects.

(d) Justification

The Watershed algorithm uses edge information and markers to accurately separate touching or merged objects, reducing under-segmentation and improving boundary detection.

(e) Conclusion

Marker-Controlled Watershed Segmentation effectively separates merged regions, improving object identification and segmentation accuracy.

25. Boundary Refinement

(a) Interpretation of the Scenario

The detected object boundaries are rough, irregular, and inaccurate due to noise or imperfect edge detection. These rough boundaries reduce the precision of image analysis and object measurement.

(b) Key Challenges in Boundary Detection

- Irregular and jagged boundaries.

- Noise affecting edge localization.

- Broken boundary segments.

- Reduced object shape accuracy.

- Difficulty in feature extraction.

(c) Suitable Refinement Method

Apply Morphological Opening and Closing Operations.

Procedure:

- Apply opening to remove small unwanted boundary irregularities.

- Apply closing to fill small gaps.

- Smooth the refined object boundaries.

- Generate the final refined boundary image.

(d) Justification

Morphological operations remove small imperfections while preserving the overall object shape. They improve boundary smoothness without significantly altering the original object.

(e) Conclusion

Morphological refinement produces smooth and accurate object boundaries, enhancing segmentation quality and improving subsequent image analysis.

26. Frequency-Based Enhancement

(a) Interpretation of the Scenario

Frequency-based enhancement improves image quality by processing the image in the frequency domain. Instead of directly modifying pixel values, the image is transformed into its frequency components, allowing selective enhancement or suppression of specific frequencies.

(b) Key Issues in Spatial Domain Methods

- Limited control over frequency components.

- Difficulty in removing periodic noise.

- Reduced effectiveness for certain image degradations.

- Simultaneous enhancement of noise and details.

- Lower flexibility for selective filtering.

(c) Appropriate Frequency Domain Technique

Apply the High-Pass Filter in the frequency domain.

Procedure:

- Compute the Fourier Transform of the image.

- Apply a High-Pass Filter to emphasize high-frequency components.

- Suppress unwanted low-frequency components.

- Perform the Inverse Fourier Transform to obtain the enhanced image.

(d) Justification

High-Pass Filtering enhances edges, fine details, and sharp transitions while reducing the influence of slowly varying background intensities. It is particularly useful for feature enhancement and edge detection.

(e) Conclusion

Frequency-domain High-Pass Filtering effectively enhances image details and sharpens edges, making the image more suitable for feature extraction, segmentation, and computer vision applications.

27. Mixed Noise Scenario

(a) Interpretation of the Scenario

The image is affected by both Gaussian noise (random intensity variations) and impulse (salt-and-pepper) noise (random black and white pixels). Since the image contains two different types of noise, using a single filtering technique may not effectively remove both while preserving image quality.

(b) Key Issues Affecting Filtering

- Presence of both Gaussian and impulse noise.

- Difficulty in selecting a single suitable filter.

- Loss of fine details during filtering.

- Edge blurring caused by excessive smoothing.

- Reduced image quality and segmentation accuracy.

(c) Suitable Combined Approach

Apply a Median Filter followed by a Gaussian Filter.

Procedure:

- Apply the Median Filter to remove impulse (salt-and-pepper) noise.

- Apply the Gaussian Filter to reduce Gaussian noise.

- Enhance the filtered image if necessary.

- Use the processed image for further analysis.

(d) Justification

The Median Filter effectively removes impulse noise while preserving edges, whereas the Gaussian Filter smooths random Gaussian noise. Combining these filters removes both types of noise without significantly degrading image details.

(e) Conclusion

Using a combination of Median and Gaussian filters effectively removes mixed noise, improves image quality, and preserves important image features for accurate processing.

28. Edge Preservation Filtering

(a) Interpretation of the Scenario

The image contains noise that must be removed without affecting important object boundaries. Preserving edges is essential because edges contain significant structural information required for feature extraction, segmentation, and object recognition.

(b) Key Issues in Filtering

- Noise reduces image quality.

- Traditional smoothing filters blur edges.

- Loss of important boundary information.

- Reduced feature extraction accuracy.

- Difficulty in maintaining image sharpness.

(c) Suitable Technique

Apply the Bilateral Filter.

Procedure:

- Define the spatial and intensity similarity parameters.

- Compute weighted averaging using neighboring pixels.

- Smooth homogeneous regions while preserving edge pixels.

- Generate the filtered image.

(d) Justification

The Bilateral Filter considers both spatial distance and intensity differences when filtering. It removes noise while preserving sharp edges, making it more effective than conventional linear filters.

(e) Conclusion

The Bilateral Filter provides excellent noise reduction while maintaining edge information, making it suitable for computer vision and image analysis applications.

29. Region Growing Failure

(a) Interpretation of the Scenario

Region Growing segmentation fails because the initial seed points are incorrectly selected. Poor seed selection causes regions to grow into incorrect areas or fail to include the complete object.

(b) Key Challenges in Region Growing

- Incorrect seed point selection.

- Noise affecting region expansion.

- Uneven intensity distribution.

- Incomplete or excessive region growth.

- Reduced segmentation accuracy.

(c) Suitable Correction

Procedure:

- Preprocess the image to remove noise.

- Automatically identify reliable seed points based on intensity or gradient.

- Apply Region Growing using similarity criteria.

- Stop growing when the threshold condition is reached.

(d) Justification

Automatic seed selection minimizes human error and ensures that the algorithm starts from appropriate object regions. This improves segmentation accuracy and prevents incorrect region expansion.

(e) Conclusion

Automatic seed selection combined with Region Growing produces accurate segmentation by correctly identifying object regions and reducing segmentation errors.

30. Industrial Inspection Enhancement

(a) Interpretation of the Scenario

An industrial inspection system must detect surface defects such as cracks, scratches, dents, or holes in noisy images. The system requires effective image enhancement and segmentation to improve defect visibility while maintaining processing accuracy.

(b) Key Issues in Detection

- Presence of image noise.

- Poor visibility of small defects.

- Edge blurring after filtering.

- Difficulty in separating defects from the background.

- Requirement for high detection accuracy.

(c) Appropriate Enhancement and Segmentation Pipeline

Pipeline:

- Apply a Median Filter to remove impulse noise.

- Apply a Bilateral Filter to reduce remaining noise while preserving edges.

- Enhance contrast using Histogram Equalization.

- Detect edges using the Canny Edge Detector.

- Perform segmentation using Otsu's Thresholding or Watershed Segmentation.

- Analyze the segmented defects for inspection.

(d) Justification

The Median Filter removes impulse noise, the Bilateral Filter preserves important edges, Histogram Equalization improves visibility, Canny accurately detects defect boundaries, and Otsu's Thresholding or Watershed Segmentation isolates defects from the background. This sequence ensures reliable defect detection with high accuracy.

(e) Conclusion

The proposed enhancement and segmentation pipeline effectively removes noise, enhances image quality, preserves critical edges, and accurately detects industrial defects. This approach improves inspection reliability, reduces false detections, and supports automated quality control.

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |