SIMATS ENGINEERING

ASSESSMENT TOOL 3: Mock Interview

COURSE NAME: COMPUTER VISION		            COURSE CODE:	ITA0518

COURSE OUTCOME COVERED

CO5: Develop computer vision applications using OpenCV for performing recognition and analysis on images and videos. (BTL 6)

Assessment Tool Weightage: 10%

Mock Interview Questions

OpenCV Basics & Pipeline Thinking

- Design and explain a complete OpenCV pipeline to read, process, and display multiple images from a folder efficiently. Clearly justify your design choices and discuss potential challenges and optimizations.

- A video file is not loading properly in OpenCV. Analyze the possible causes and propose a structured debugging approach, explaining each step logically.

Feature Detection & Drawing

10.You need to compare two images and highlight differences visually. Analyze the problem and design a solution, justifying your approach.

Face & Expression Recognition

- A face detection system produces multiple false positives. Analyze the issue and propose improvements, explaining how each change enhances accuracy.

- You need to build a simple face recognition demo. Design a minimal working pipeline, explaining each stage and your design decisions.

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Technical Knowledge | 25 | Demonstrates strong and accurate subject knowledge with in-depth understanding | Shows good knowledge with minor gaps | Basic knowledge with noticeable errors | Lacks fundamental technical knowledge |
| Problem Solving | 25 | Solves problems logically and applies concepts effectively in real scenarios | Applies concepts with moderate accuracy | Limited problem-solving ability; requires guidance | Unable to solve or apply concepts |
| Analytical Thinking | 15 | Analyzes questions critically and provides well-structured, logical answers | Provides reasonable analysis with some structure | Superficial or partially structured responses | No analytical thinking demonstrated |
| Communication | 15 | Communicates clearly, confidently, and professionally with proper terminology | Communicates well with minor hesitation | Communication lacks clarity or confidence | Unable to communicate effectively |
| Confidence | 15 | Displays high confidence, positive attitude, and professional behavior throughout | Shows good confidence with minor issues | Low confidence; inconsistent behavior | Lacks confidence and professionalism |
| Response to Questions | 10 | Responds accurately, promptly, and elaborates with relevant examples | Responds correctly but with limited depth | Partial responses; needs prompting | Unable to respond appropriately |

ANSWERS

The main limitation is that the expression recognition system is sensitive to changes in face orientation. When the person moves their head slightly, important facial features such as the eyes, eyebrows, and mouth may shift from their expected positions.

Proposed modifications:

- Use a robust face detector to detect the face in every frame.

- Apply face alignment using facial landmarks.

- Normalize the face orientation before expression recognition.

- Use image preprocessing such as resizing and illumination normalization.

- Train the recognition model using images with different head poses.

- Use tracking between frames to maintain stable face locations.

These modifications make the system more robust to small head movements and improve expression recognition accuracy.

8.

The objective is to avoid saving every video frame and store only frames where there is a significant visual change.

Method:

- Read the video frame by frame using cv2.VideoCapture().

- Convert each frame to grayscale.

- Resize the frame if necessary to reduce computation.

- Compare the current frame with the previously saved frame.

- Calculate the difference using absolute frame subtraction.

- Apply a threshold to identify significant changes.

- Calculate the percentage of changed pixels.

- Save the frame only when the percentage exceeds a predefined threshold.

Decision criterion:

If the calculated difference is greater than the selected threshold, the frame is considered significantly different and is saved.

This approach reduces storage requirements and processing time while preserving important changes in the video.

5. Frame dropping occurs when the processing time for each frame is greater than the time available between consecutive video frames. Heavy image processing, large frame sizes, slow hardware, or inefficient code can cause this problem.

Improved approach:

- Reduce the input frame resolution.

- Process only the required frames instead of every frame.

- Use efficient OpenCV functions.

- Separate frame capture and processing using a buffer or separate thread.

- Use lightweight algorithms for real-time processing.

- Avoid unnecessary image copying.

- Display only the processed output required by the application.

Trade-off:
Reducing resolution or skipping frames improves speed but may reduce image quality or detection accuracy. Therefore, the processing level should be selected according to the application's requirements.

This redesigned pipeline provides smoother playback while maintaining acceptable processing accuracy.

13.

A simple face recognition system can be designed using the following pipeline:

Input Image/Video → Face Detection → Face Preprocessing → Feature Extraction → Face Recognition → Display Result

Stages:

- VInput: Capture an image or video frame using OpenCV.

- Face Detection: Detect faces using a suitable face detector.

- Preprocessing: Crop the detected face, resize it, and normalize the image.

- Feature Extraction: Extract important facial features from the face image.

- Recognition: Compare the extracted features with stored face data.

- Output: Display the recognized person's name and confidence score using OpenCV drawing functions.

For a simple demo, a small number of known faces can be stored as reference images. The system should also handle cases where no face is detected.

This pipeline is simple, modular, and can be extended later for multiple faces and real-time recognition.

3.

The solution is to capture frames continuously from a webcam, process each frame, and overlay the required text and shapes before displaying it.

Pipeline:

Webcam → Capture Frame → Draw Text/Shapes → Display Frame → Repeat

Implementation steps:

- Use cv2.VideoCapture() to access the webcam.

- Read each frame using cap.read().

- Use cv2.putText() to add text.

- Use drawing functions such as cv2.rectangle(), cv2.circle(), and cv2.line() to add shapes.

- Display the modified frame using cv2.imshow().

- Use cv2.waitKey() to control the display and detect a key for exiting.

- Release the camera using cap.release() and close windows using cv2.destroyAllWindows().

For efficiency, unnecessary processing should be avoided and the frame resolution can be reduced if required. Drawing directly on the current frame also avoids unnecessary memory usage.

This provides a simple and efficient real-time overlay system using OpenCV.